ClarkDietrich® Exterior Framing

Section 070543 Cladding Support Systems

NOTE:

Turn on hidden text to view important non-printing notes by selecting Show/Hide (¶) in the Paragraph Group, or by using the Ctrl+* keyboard shortcut.

- GENERAL

      - SUMMARY

Retain "Related Requirements" Paragraph below to cross-reference requirements Contractor may expect to find in this Section but are specified elsewhere. These may include, but are not limited to: “Section 033000 – Cast-in-Place Concrete,” “Section 042600 – Single-Wythe Unit Masonry,” “Section 054000 – Cold-Formed Metal Framing,” “Section 061600 – Sheathing,” “Section 070500 – Common Work Results for Thermal and Moisture Protection,” “Section 072100 – Thermal Insulation,” “Section 072500 – Weather Barriers,” “Section 072600 – Vapor Retarders,” “Section 072700 – Air Barriers,” “Section 074200 – Wall Panels,” “Section 092900 – Gypsum Board,” or related Sections.

Coordinate with Related Requirements in applicable Division 01 Sections.

        - Related Requirements:

          - <Insert Section Number and Title>.

      - INFORMATIONAL SUBMITTALS

Retain “Informational Submittals” Paragraph below if NFPA 285 compliance is required.

        - Technical Evaluation Report (TER): Submit third-party Technical Evaluation Report demonstrating cladding support system’s regulatory compliance with NFPA 285.

      - QUALITY ASSURANCE

Retain “Vertical and Lateral Fire Propagation Test Characteristics” Paragraph below if NFPA 285 compliance is required.

        - Vertical and Lateral Fire Propagation Test Characteristics:

          - Exterior Wall Assemblies: Tested in accordance and complying with NFPA 285 acceptance criteria.

            - Evaluate base wall, stud cavity insulation, wall sheathing, air/water barrier, continuous insulation, cladding support system, and exterior cladding as part of assembly test.

        - DELIVERY, STORAGE, AND HANDLING

        - Protect cold-formed metal framing from corrosion, deformation, and other damage during delivery, storage, and handling as required by AISI S202.

- PRODUCTS

      - PERFORMANCE REQUIREMENTS

        - Cold-Formed Steel Framing Standards: Unless more stringent requirements are indicated, framing to comply with AISI S100 and AISI S240.

        - Design wall systems in consideration of thermal expansion and contraction effects. Include thermal breaks in accordance with pertinent codes and regulations of Authorities Having Jurisdiction.

      - CONTINUOUS INSULATION RAINSCREEN FRAMING SYSTEM

        - Steel Sheet: ASTM A1003/A1003M, Structural Grade, Type H, metallic coated, of grade and coating designation as follows:

          - Grade: ST50H (ST340H).

          - Coating: [CP 90: G90 (Z275), AZ50 (AZM150), or GF45 (ZGF135)] [CP 90: G90 (Z275), AZ50 (AZM150), or GF45 (ZGF135) with black powder coating].

        - Vented Rainscreen Attachment System: Hat-shaped channel for exterior wall cladding attachment, with pre-punched holes for fastening and vent holes for moisture drainage and air movement. Hat-shaped channel is attached over [foam plastic] [mineral wool] [spray polyurethane foam] insulation using manufacturer’s tested thermally-isolated fasteners.

          - Product: ClarkDietrich; ProChannel Ci Cladding Support with Grip-Deck TubeSeal® Technology or comparable product.

          - Minimum Base-Steel Thickness: 0.0538 inch (1.37 mm).

          - Depth: 7/8 inch (22 mm).

          - Orientation: [Horizontal] [Vertical].

      - ACCESSORIES

        - Mechanical Fasteners:

          - ASTM C 1513; manufacturer’s recommended thermally-isolated self-tapping, steel drill screws with integral sleeve, for fastening vented rainscreen attachment system to [cold-formed steel] [wood] structural framing.

            - Fastener Depth: Coordinate with thickness of insulation required.

- EXECUTION

      - EXAMINATION

        - Examine substrates, areas, conditions, and abutting structural framing for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Ensure air/water barrier is fully installed and cured in accordance with manufacturer’s instructions and Project requirements.

        - Flash and seal all fenestration, transitions, discontinuities, sills, and ledgers in accordance with manufacturer’s instructions and Project requirements.

        - Refer to manufacturer’s structural span tables for recommended channel and fastener spacing.

      - INSTALLATION OF VENTED RAINSCREEN ATTACHMENT SYSTEM

        - Install vented rainscreen attachment system over studs and continuous insulation, in accordance with ASTM C1007, AISI S240 and manufacturer’s written instructions, unless more stringent requirements are indicated.

        - Fasten channels through insulation and sheathing, into structural studs using manufacturer’s recommended thermally-isolated fasteners.

        - Comply with requirements for spacing, edge distances and minimum penetration depth.